Algorithm for Screw and Reciprocating Compressors

Outline

- Flowchart

- Types of Compressors and Control Types

- Common key

- Calculations common to all compressors

  - Pressure Correction

  - Altitude Correction

- Finding Baseline Compressors Parameters

- Performance Curves

  - Section 1: Finding Performance Curves for Individual Compressor under Load/Unload Controls

    - Compressor Cycling Model

    - Compressor Cycling Model – Constants

    - Deriving Performance Curves using the Compressors Cycling Module

  - Section 2: Finding Performance Curves for Individual Compressor under Modulating Controls

    - Modulation without unloading

  - Section 3: Finding Performance Curves for Individual Compressor under Hybrid controls (Variable displacement with unloading)

  - Section 4: Finding Performance Curves for Individual Compressor under start/stop

Contents

Calculations – Flowchart	3

Types of Compressors and Control Types	4

Finding Baseline Compressors Parameters (Transforming user inputs)	6

Finding Performance Curves	7

Section 1: Finding Performance Curves for Individual Compressor under Load/Unload Controls	7

Step 1: Adjusting Performance Points (separate algorithm)	9

Step 2: Choose appropriate default values based on type of compressor	10

Step 3: Pre-Model Calculation Set	11

Deriving Performance Curves using the Compressors Cycling Module	15

Section 2: Finding Performance Curves for Individual Compressor under Modulating Controls without Unloading	16

Step 1: Adjusting Performance Points (inputs)	16

Step 2: Calculating Power/Capacity (Performance profile)	17

Section 3: Finding Performance Curves for Individual Compressor under Modulating Controls with Unloading	19

Section 4: Finding Performance Curves for Individual Compressor under Start/Stop controls	20

Step 1: Adjusting Performance Points (inputs)	21

Step 2: Calculating Power/Capacity (Performance profile)	21

Compressor Information

(Non-Centrifugal)

Generic and under input

Control Type?

Adjust for Inlet conditions and setting changes if adjusted by user

(discharge psi, altitude, inlet psi)

User Operation (profile) Input

(logtool, data explorer, hourly data)

Transform User Input if needed

(ex. converting V,I and PF to kW)

Load/Unload

(section 1)

Calculate equations using 76pt from compressor settings

Calculate Baseline Using Equations

Calculate equations using 76pt from compressor settings – same as load/unload (section 1)

Find point and switch to modulation equations (section 2)

0

From Database and User

Inlet condition Document

Non-Centrifugal Doc (this doc)

Modulation without Unloading

(section 2)

Start/Stop

(section 4)

Modulation with Unloading

Or Variable Displacement

(Section 3)

Calculations – Flowchart

# Types of Compressors and Control Types

Airmaster can model the six different types of compressors and allow 12 unique control types. Not all compressors can work with any control type however and the Compressor Type/ Control Type pairs are listed below.

| Compressor Types | Associated Controls Types |
| --- | --- |
| Single stage lubricant-injected rotary screw | Inlet modulation without unloading Inlet modulation with unloading Variable displacement with unloading Load/unload VFD (not in document yet) |
| Two stage lubricant-injected rotary screw | Inlet modulation without unloading Inlet modulation with unloading Variable displacement with unloading Load/unload VFD (not in document yet) |
| Two stage lubricant-free rotary screw | Load/unload VFD (not in document yet) Start/Stop |
| Single stage reciprocating | Load/unload Start/Stop |
| Two stage reciprocating | Load/unload Start/Stop Multi-step unloading |
| Multiple Stage Centrifugal | Load/unload Inlet butterfly modulation with blowoff Inlet butterfly modulation with unloading Inlet guide vane modulation with blowoff Inlet guide vane modulation with unloading |

Common Key

| Inputs - from user and database |  | Units | Imperial Units | Metric Units |
| --- | --- | --- | --- | --- |
| Compressor Rated capacity (acfm): | C_fl | acfm | acfm |  |
| Full Load (cut-in) Pressure (psig): | P_fl | psig | psig |  |
| Max Full Flow Pressure (psig): | P_max | psig | psig |  |
| Power at Full Load Operating Pressure (kW): | kW_fl | kW | - | kW |
| Power at Max Full Flow Operating Pressure (kW): | kW_max | kW | - | kW |
| Load Factor Fully Modulated: | lf_fl | % (fraction) | - | - |
| Proportional Modulating Pressure Range (full range, psi) | P_mod | psig | psig |  |
| Unload Pt. (%capacity, use 100 for load/unload): | PerC_ul | % (fraction) | - | - |
| Atmospheric Pressure (psia): | P_atm | acfm | acfm |  |
| Air Usage | C_usage | acfm | acfm |  |
| Power usage | kw_usage | kW | - | kW |
| Receiver Volume (cu ft) | C_storage | acfm | acfm |  |

# Finding Baseline Compressors Parameters (Transforming user inputs)

Any compressor modelled in the tool can take in any one of the following baseline parameter, entered an hourly basis by the user for each daytype (via logtool/ Data Explorer), and calculate the other baseline parameters from it.

These inputs “Profile” can be any of following

- Power in kW

- Airflow, % Capacity

- Airflow, acfm

- Cycle Time

- Volts/Amp

In is important to know all the above parameters for the baseline. This section discusses how to find these given an given “Profile”.

| Input \ Output | Power | % Capacity | Acfm |
| --- | --- | --- | --- |
| Power | Same as input | - Find instantaneous %power from Max Power - Find % Capacity from performance curves | Acfm = Acfm_max x %capacity |
| % Capacity | - Find % Power from performance curves - Power = P_max x %Power | Same as input | Acfm = Acfm_max x %capacity |
| Acfm | - Find % Power from %P vs %C chart - Power = P_max x %Power | %Capacity=AcfmAcfm_max | Same as input |
| Volts/ Amps | Power = 3x Volts x Amps x power factor | - Find instantaneous %power from Max Power - Find % Capacity from performance curves | Acfm = Acfm_max x %capacity |
| Cycle Time |  |  |  |

Irrespective of the compressor under consideration and the control type it follows, this common structure will help determine all the baseline parameters needed for further analysis. The relationship between the baseline paraments, establised by its performance curve, is vastly different for each compressor however and the method used to derive it is detailed in the following section.

# Finding Performance Curves

## Section 1: Finding Performance Curves for Individual Compressor under Load/Unload Controls

As seen in the table above, the performance curves are essential to calculate the baseline performance of a compressor. This section details how the performance of a screw and reciprocating compressors operating under load/unload control type is determined for any given  user input.

Applicable Key

| Inputs |  | Units | Imperial Units | Metric Units |
| --- | --- | --- | --- | --- |

| Performance Points – Inputs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Full Load (cut in) – Discharge Pressure | P_fl | psig | psig | - |
| Full Load (cut in) – Airflow | C_fl | acfm | acfm | - |
| Full Load (cut in) – Power | kw_fl | kW | - | kW |
| Max Full Flow (cut-out) - Discharge Pressure | P_max | psig | psig | - |
| Max Full Flow (cut-out) – Airflow | C_max | acfm | acfm |  |
| Max Full Flow (cut-out) – Power | Kw_max | kW | - | kW |
| No Load (unloaded) - Discharge Pressure | P_nl | psig | psig | - |
| No Load (unloaded) - Airflow | C_nl | acfm | acfm | - |
| No Load (unloaded) – Power | kw_nl | kW | - | kW |

| Inlet Conditions/ System Inputs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Average Temperature | T_amb | psig | psig | - |
| Atmospheric Pressure (psia): | P_atm | acfm | acfm |  |
| Receiver Volume (cu ft) | C_storage | acfm | acfm |  |

| Calculated Values |  |  |  |  |
| --- | --- | --- | --- | --- |
| Air Usage | C_usage | acfm | acfm |  |
| Proportional Modulating Pressure Range (full range, psi) | P_mod | psig | psig |  |
| Load Factor Unloaded: | lf_nl | % (fraction) | - | - |
| Calculated Fully Modulated from Full Load Power (kW): | kw_flmod | kW | - | kW |
| Calculated Fully Modulated from Max Full Flow Power (kW): | kw_maxmod | kW | - | kW |
| Calculated Power at Unload Point (kW): | kw_ul | kW | - | kW |
| Calculated Airflow at Unload Point (acfm): | C_ul | acfm | acfm |  |
| Modulation Exponent (1=throttle, 2=variable displacement) | mod_exp | no unit | - | - |
| Unload (cut-out) Pressure (psig): | P_ul | % (fraction) | - | - |
| Blowdown Time Constant (sec): | t_bdc | sec | - | - |
| Suction Pressure Time Constant During Unloading: | t_spc |  |  |  |
| Receiver Drawdown (sec) | t_dd | sec |  |  |
| Blowdown (sec) | t_bd | sec |  |  |
| Achieved Blowdown Sump Pressure (psig) | P_sump_bla |  |  |  |
| Min. Load During Blowdown | kw_min_bd | kW | - | kW |
| Avg. Load During Blowdown | kw_avg_bd | kW | - | kW |
| Offload (sec) | t_ol |  |  |  |
| Load during offload | kw_avg_ol | kW | - | kW |
| Compressor Reload (sec) | t_rl |  |  |  |
| Avg. Load During Reload | kw_avg_rl | kW | - | kW |
| Receiver Pumpup to Max. Full Flow (sec) | t_rpu |  |  |  |
| Avg. Load During Receiver Pumpup | kw_avg_rpu | kW | - | kW |
| Receiver Pumpup During Modulation (sec) | t_rmod |  |  |  |
| Avg. Discharge Pressure During Modulation (psig) | P_avg_mod |  |  |  |
| Avg. Load During Modulation | kw_avg_mod | kW | - | kW |
| Total Time Cycle (sec) | T_cycle |  |  |  |
| Average Load (kW) for curve fit | kw_curve | kW | - | kW |
| Average capacity (acfm) for curve fit | C_curve | acfm | acfm |  |
| Power at blowdown | kw_bd | kW | - | kW |
| Power at offload | kw_ol | kW | - | kW |
| Power at reload | kw_rl | kW | - | kW |
| Power at receiver pump up to max full flow | kw_rpu | kW | - | kW |
| Power at receiver pump up during modulation | kw_rmod | kW | - | kW |
| Percentage of capacity for curve fit | PerCap | % (fraction) |  |  |
| Percentage of power for curve fit | Perpow | % (fraction) |  |  |
| Percentage of capacity calculated | PerC | % (fraction) |  |  |
| Percentage of power calculated | Perkw | % (fraction) |  |  |
| Capacity calculated | C_calc | acfm | acfm |  |
| Power calculated | kw_calc | kW | - | kW |

| Constants – Default values |  | Units | Imperial Units | Metric Units |
| --- | --- | --- | --- | --- |
| Unloaded Sump Pressure (psig): | P_sump | psig | psig | - |
| Time to Blowdown within Approach Tolerance (sec): | t_blowdown | sec | - | - |
| Suction Develop Time within Approach Tolerance (sec): | t_sdt | sec | - | - |
| Approach Tolerance for Unloading and Suction: | a_tol |  |  |  |
| Time to Reload (sec): | t_reload | sec | acfm |  |
| Unload Pt. (%capacity, use 100 for load/unload): | PerCul | % (fraction) | - | - |
| Load Factor Fully Modulated: | lf_fl | % (fraction) | - | - |

Compressor Cycling Model – Algorithm

The following section lays out the algorithm for the compressor cycling model. This model will be used to compute, with adjustments, the performance curves for screw and reciprocating compressors operating under the load and unload controls. Step 1 summarizes the pre-calculation adjustments related to pressure and inlet conditions, Step 2 outlines the default values to be used for the different compressors types and the core algorithm is discussed in Steps 3 and 4.

### Step 1: Adjusting Performance Points (separate algorithm)

Perform the appropriate corrections as laid out in the “Pressure and Inlet Correction” section of the algorithm. Summarizing the changes that will affect the modulating controls below.

| How power and airflow changes with discharge pressure | Discharge Pressure | Airflow | Airflow | Power | Power |
| --- | --- | --- | --- | --- | --- |
| Full Load (cut in) | P_fl | C_fl | Will Change | kW_fl | Will Change |
| Max Full Flow (cut-out) | P_max | C_max | Will Change | kW_max | Will Change |
| No Load (unloaded) | P_nl | C_nl = 0 | Can't Change | kW_nl | Will not change |

### Step 2: Choose appropriate default values based on type of compressor

The default values used for calculating the performance of a screw compressors operating in load/unload control will depend on the type of compressor.

Single Stage and Two Stage Lubricant Injected Screw Compressors – Load/Unload Controls

| Constants |  |  |
| --- | --- | --- |
| Unloaded Sump Pressure (psig): | P_sump | 15 |
| Time to Blowdown within Approach Tolerance (sec): | t_blowdown | 20 |
| Suction Develop Time within Approach Tolerance (sec): | t_sdt | 2.0 |
| Approach Tolerance for Unloading and Suction: | a_tol | 0.02 |
| Time to Reload (sec): | t_reload | 3 |
| Unload Pt. (%capacity, use 100 for load/unload): | PerCul | 1 |
| Load Factor Fully Modulated: | lf_fl | 0.7 |

Two Stage Lubricant Free Screw Compressors – Load/Unload Controls

| Constants |  |  |
| --- | --- | --- |
| Unloaded Sump Pressure (psig): | P_sump | 15 |
| Time to Blowdown within Approach Tolerance (sec): | t_blowdown | .003 |
| Suction Develop Time within Approach Tolerance (sec): | t_sdt | .004 |
| Approach Tolerance for Unloading and Suction: | a_tol | 0.02 |
| Time to Reload (sec): | t_reload | .001 |
| Unload Pt. (%capacity, use 100 for load/unload): | PerCul | 1 |
| Load Factor Fully Modulated: | lf_fl | 0.7 |

Single Stage and Two Stage Reciprocating Compressors – Load/Unload Controls

| Constants |  |  |
| --- | --- | --- |
| Unloaded Sump Pressure (psig): | P_sump | 15 |
| Time to Blowdown within Approach Tolerance (sec): | t_blowdown | .003 |
| Suction Develop Time within Approach Tolerance (sec): | t_sdt | .004 |
| Approach Tolerance for Unloading and Suction: | a_tol | 0.02 |
| Time to Reload (sec): | t_reload | .001 |
| Unload Pt. (%capacity, use 100 for load/unload): | PerCul | 1 |
| Load Factor Fully Modulated: | lf_fl | 0.7 |

-----------------------------------------------------------------------------------------------------------

### Step 3: Pre-Model Calculation Set

These following values needs to be calculated prior to running the compressors cycling model. We will call this set of equations the “pre-model calculation set” as we will be needing to call this specific set of steps later.

Load Factor Unloaded ,                                                                      lfnl =kwnl kwfl

Calculated Fully Modulated from Full Load  Power  ,               kwflmod=lffl . kwfl

Calculated Fully Modulated from Max Full Flow Power ,  kwmaxmod=lffl . kwmax

Calculated Airflow at Unload Point acfm ,                          Cul= Cfl.  PerCul

Calculated Power at Unload Point ,  kwul=kwmax-kwmaxmod  . CulCflxmod+kwmaxmod

Unload cut-outPressure psig,                                            Pul=Pmax+1- CulCfl . Pmod

Blowdown Time Constant sec,                                       tbdc=tblowdownlog⁡1atol

Suction Pressure Time Constant During Unloading,      tspc=tsdtlog⁡1atol

-------------------------------------------------------------------------------------------------------------------------------

Compressor Cycling Model - Finding Average Power (kwcurve)

If Ccurve<Cul   then ,  kwcurve=kwbd+kwol+kwrl+kwrpu+kwrmod   Tcycle

Else ,         kwcurve=kwavgmod

Note: Ccurve is determined as laid out at the end of this section. Refer “Deriving Performance Curves using the Compressors Cycling Module” at the end of Section 1.

The average power is calculated by adding the power drawn by the compressors during its different stages of its operations cycle. The power drown at the individual stages are calculated as below,

- Finding power at blowdown (kwbd)

kwbd=tbd . kwavgbd

Finding time to blowdown (tbd)

tbd=min(tblowdown, tdd )        ;  minimum of tblowdown and tdd

Time to drawdown,   tdd=Cstorage. 60 .Pul-Pfl  Ccurve. Patm

Finding average power at blowdown (kwavg_bd)

kwavg_bd= kwnl +kwul-kwmaxmod. e-tbdtspc +kwmaxmod-kwnl 1-e-tbdtbdc. tbdctbd

Finding minimum power at blowdown (kwmin_bd)

If            tbd=tblowdown   then,  kwmin_bd = kwnl

Else,

kwmin_bd = kwnl  + kwul-kwmaxmod. e-tbdtspc +kwmaxmod-kwnl . e-tbdtbdc

- Finding power at offload (kwol)

kwol=tol . kwavg_ol

Finding time to offload (tol)

tol=tdd-tbd

Finding average power at offload (kwol)

kwavg_ol= kwfl.  lfnl

3. Finding power to reload compressor (kwrl)

kwrl=trl . kwavg_rl

Finding time to reload compressor (trl)

trl=treload .  Pul - PsumpblaPul-Pul-Psumpul. atol

Where, Psumpbla= Psumpul+Pul-Psumpul× exp-tbdtbdc

Finding average power to reload compressor (kwrl)

kwavg_rl=kwmin⁡_bd+kwfl2

4. Finding power to Pump up receiver at max full flow (kwrpu)

kwrpu=trpu . kwavg_rpu

Finding time for receiver pump-up to max full flow (trpu)

trpu=60. CstoragePmax-Pfl +trl60 . Patm.Ccurve  Cstorage  Patm (Cfl-Ccurve).

Finding average power for receiver pump-up at max full flow (kwavg_rpu)

kwavg_rpu=1-Patm.  Ccurve.  trlCstorage .60 .200. kwfl+kwmax2

5. Finding power to Pump up receiver during modulation (kwrmod)

If Cul<>Cfl   then ,  kwrmod=trmod . kwavg_mod

Else ,         kwrmod=0

Finding Time for Receiver Pump-up During Modulation (trmod)

If Ccurve<Cul  and Cul<>Cfl  then,    trmod= Pmod .Cstorage Patm . Cfl  logCfl-Ccurve Cul-Ccurve. 60

Else ,  trmod=NA

Finding Avg. Discharge Pressure During Modulation (Pavg_mod)

If Ccurve<Cul  and Cul<>Cfl then,

Pavg_mod= Pmax+ Pmod-Ccurve . PmodCfl   +Ccurve  .  PmodCfl-Prange  .

.PmodPatm.CstorageCfl .1-exp-PatmPmod.Cfl Cstorage .trmod60trmod.60

Else ,     Pavg_mod=NA

6. Finding Average power during modulation (kwavg_mod)

If Cul<>Cfl  then,

if Ccurve<Cul then

kwavg_mod=  (kwmax- kwmaxmod) . Pmax +Pmod -Pavgmod Pmodxmod+kwmaxmod

Else, kwavg_mod=  (kwmax- kwmaxmod) . CcurveCfl  xmod+kwmaxmod

Else ,  kwmod= NA

### Deriving Performance Curves using the Compressors Cycling Module

If flow is the input calculate the results directly, if kW is the input use a curve fit;

Step 1 – Divide the capacity of compressor(s) (C_fl) (cfm) into 76 bins. Individual ‘capacity’ points associated with each bin are termed C_curve.

Step 2 – For each bin calculate power (kwcurve) using the equations above

Step 3 – Calculate PerPow and PerCap for each bin.

PerPow=kwcurvekwfl       ;   PerCap=CcurveCfl

Example: If rated capacity of compressors (Cfl) is 473 cfm. Then 76 bins are created and power is calculated as such

|  | Bin 1 | Bin 2 | Bin 3 | Bin4 | Bin 5 | …. | Bin 74 | Bin 75 | Bin 76 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C_curve | 473 | 467 | 460 | 454 | 448 | …. | 13 | 6 | 0 |
| perCap | 100% | 99% | 97% | 96% | 95% | …. | 3% | 1% | 0% |
| kw_curve | 85.4 | 86.83 | 86.00 | 85.17 | 84.33 | …. | 18.28 | 17.19 | 16.10 |
| perPow | 100% | 102% | 101% | 100% | 99% | …. | 21% | 20% | 19% |

Step 4 – A polynomial curve fit of the 4th order is generated from the 76 data points using perCap and perPow and vice versa. Comment: use Curve fit from pumps

Step 5 – Calculate the required parameter from the user input (from data explorer) using appropriate polynomial equation.

The user input can either be in Cusage, in which can kwcalc is to be found from the polynomial equation. The user input can also be in terms of kwusage , in which case the Ccalc is the unknown which needs to be estimated.

## Section 2: Finding Performance Curves for Individual Compressor under Modulating Controls without Unloading

This algorithm applies to single stage and two stage lubricant-injected rotary screw compressors operating under “Modulation” controls without unloading

Applicable Key

| Inputs |  | Units | Imperial Units | Metric Units |
| --- | --- | --- | --- | --- |

| Performance Points – Inputs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Full Load – Discharge Pressure | P_fl | psig | psig | - |
| Full Load – Airflow | C_fl | acfm | acfm | - |
| Full Load – Power | kw_fl | kW | - | kW |
| No Load (modulated) - Discharge Pressure | P_nl | psig | psig | - |
| No Load (modulated) - Airflow | C_nl | acfm | acfm | - |
| No Load (modulated) – Power | kw_nl | kW | - | kW |

| Inlet Conditions – Inputs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Average Temperature | T_amb | psig | psig | - |
| Atmospheric Pressure (psia): | P_atm | acfm | acfm |  |

| Others |  |  |  |  |
| --- | --- | --- | --- | --- |
| Load Factor Fully Modulated: | lfnl |  | - | - |
| Modulation Exponent (1=throttle, 2=variable displacement) | xmod | - | - | - |
| Percent Power Drawn | Perkw | % |  |  |
| Percent Capacity | PerC | % |  |  |
| Power drawn at given point | kwavg | kW | - | kW |
| Airflow at given point | Cusage | acfm | acfm | - |
| Capacity Calculated | Ccalc | kW | - | kW |
| Power Calculated | kwcalc | acfm | acfm | - |

### Step 1: Adjusting Performance Points (inputs)

Perform the appropriate corrections as laid out in the “Pressure and Inlet Correction” document (separate doc). Summarizing the changes that will affect the modulating controls below.

| How power and airflow changes with discharge pressure | Discharge Pressure | Airflow | Airflow | Power | Power |
| --- | --- | --- | --- | --- | --- |
| Full Load | P_fl | C_fl | Will Change | kW_fl | Will Change |
| No Load (modulated) | P_nl (will change with P_fl based on compressor settings (must keep modulation range) one way only | C_nl = 0 | Can't Change | kW_nl | Will not change with nl, will change with fl |

### Step 2: Calculating Power/Capacity (Performance profile)

Load factor,               lfnl =kwnl kwfl

Modulation Exponent,     mod_exp=1

If Capacity is the Input

Cusage for each individual input is given by logtool analysis; other inputs are provided by the user.

kwavg=kwfl- kwnl × CusageCflmod_exp+kwnl

Ccalc=Cusage

kwcalc=kwavg

If power is the Input

kwavg for each individual bin is given by logtool analysis; other inputs are provided by the user.

Perkw=kwavgkwfl

If Perkw<lfnl   then ,  Cusage=0

Elseif  ,       Perkw≥1     Cusage=Cfl

Else ,         Cusage=kwavg- kwnlkwfl- kwnl 1mod_exp× Cmax

Ccalc=Cusage

kwcalc=kwavg

## Section 3: Finding Performance Curves for Individual Compressor under Modulating Controls with Unloading

The following types of compressors and associated controls covered in this section of the algorithm are tabulated below.

| Compressor Types | Associated Controls Types |
| --- | --- |
| Single stage lubricant-injected rotary screw | - Inlet modulation with unloading - Variable displacement with unloading |
| Two stage lubricant-injected rotary screw | - Inlet modulation with unloading - Variable displacement with unloading |

“Inlet Modulation with Unloading” and “Variable Displacement with unloading” are both hybrid control types. In these controls the compressors operate as a modulating machine above a designated “Unload Point” and operates under load/unload control strategy below the control point.

Applicable Key

| Inputs |  | Units | Imperial Units | Metric Units |
| --- | --- | --- | --- | --- |

| Performance Points – Inputs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Full Load – Discharge Pressure | P_fl | psig | psig | - |
| Full Load – Airflow | C_fl | acfm | acfm | - |
| Full Load – Power | kw_fl | kW | - | kW |
| Max Full Flow – Discharge Pressure | P_max | psig | psig | - |
| Max Full Flow – Airflow | C_max | acfm | acfm | - |
| Max Full Flow – Power | kW_max | kW | - | kW |
| Unload Point – Discharge Pressure | P_ul | psig | psig | - |
| Unload Point – Airflow | C_ul | acfm | acfm | - |
| Unload Point – Power | Kw_ul | kW | - | kW |
| No Load (modulated) - Discharge Pressure | P_nl | psig | psig | - |
| No Load (modulated) - Airflow | C_nl | acfm | acfm | - |
| No Load (modulated) – Power | kw_nl | kW | - | kW |

| Others |  |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation Exponent (1=throttle, 2=variable displacement) | mod_exp | - | - | - |
| Unload point | PerUL | % |  |  |
| Load Factor Unloaded: | lf_nl | % (fraction) | - | - |
| Load Factor fully modulated: | lf_fl | % (fraction) | - | - |
| Calculated Fully Modulated from Full Load Power (kW): | kw_flmod | kW | - | kW |
| Calculated Fully Modulated from Max Full Flow Power (kW): | kw_maxmod | kW | - | kW |

This table only shows the new variables application to the Inlet modulation with unloading and Variable displacement with unloading control types. In addition, other legacy variables are grandfathered in from Load/unload and Modulating controls types

The calculations steps can be summarized as follows,

### Step 1: Adjusting Performance Points (inputs)

- The default performance point is populated by the database values

- If user changes Unload point (PerUL) ; the following are edited in the performance points

  - Cul= PerUL x Cfl

  - Kw_ul  as per power – airflow relationship

  - P_ul based on discharge pressure- power/airflow relationship

Similarly, perform the appropriate corrections as laid out in the “Pressure and Inlet Correction” document (separate doc). Summarizing the changes that will affect the modulating controls below.

| How power and airflow changes with discharge pressure | Discharge Pressure | Airflow | Airflow | Power | Power |
| --- | --- | --- | --- | --- | --- |
| Full Load (cut in) | P_fl | C_fl | Will Change | kW_fl | Will Change |
| Max Full Flow (mod-begins) | P_max | C_max | Cant change | kW_max | Cant change |
| Unload point (cut-out) | P_ul | C_ul | Will Change | Kw_ul | Will Change |
| Noload (unloaded) | P_nl | C_nl = 0 | Can’t Change (value - 0) | kW_nl | Will not change |

### Step 2: Calculating Power/Capacity (Performance profile)

“Inlet Modulating with Unloading” and  “Variable Displacement with Unloading” control types follow the same set of equations with the only difference being in the modulating exponent (mod_exp) being used. This is a constant value and is 1 for inlet modulating and 2 for variable displacement.

Modulation Exponent,  mod_exp=1  for “Inlet Modulating with Unloading”

mod_exp=2  for “variable displacement with Unloading”

lffl=0.7

kwmaxmod = lffl × kwmax

If Capacity is the Input

Cusage for each individual bin is given by logtool analysis; other inputs are provided by the user.

If Cusage >= C_ul;  (compressor operating in modulation mode)

If Cusage = Cfl   then ,  kwavg=kwfl

Else ,  kwavg=kwmax- kwmaxmod × CusageCflmod_exp+kwmaxmod

Ccalc=Cusage

kwcalc=kwavg

Note: While these modulation equations are similar to how we calculate for inlet modulation (section 2) ; there are noticeable differences

- The value for fmod_exp, which depends on what the user chooses for control type ( 1 for inlet modulation / 2 for variable displacement)

- The introduction of additional operating point “Max Full Flow (mod-begin point)” changes the equations

If Cusage<  C_ul;  (compressor operating in load/unload mode)

- Calculate power similar to how it is calculate for from load/unload  (section 1)

If power is the Input

kwavg for each individual bin is given by logtool analysis; other inputs are provided by the user.

If kwavg >= kw_ul;  (compressor operating in modulation mode)

Perkw=kwavgkwfl

If kwavg=kwfl  then ,    Cusage = Cfl

Else ,         Cusage=kwavg- kwmaxmodkwmax- kwmaxmod 1mod_exp× Cfl

Ccalc=Cusage

kwcalc=kwavg

If kwavg < kw_ul ;  (compressor operating in load/unload mode)

- Calculate capacity similar to how it is calculate for from load/unload  (section 1)

- Divide the capacity of compressor(s)into bins to derive a curve fit equation as defined in the “ “Deriving Performance Curves using the Compressors Cycling Module” – The bins are divided from capacity at no load (Cul)  to unload capacity (Cul) (the unload point itself is not included) itself. Note: The capacity at full load (Cfl)  does not play a role in dividng the bins as it does in section 1.

- Example: If full load capacity of compressors (Cfl) is 237 cfm, unload capacity (Cul) is 116 and no capacity (Cnl) is 0. Then the 76 bins are created between 0 to 115.9, and power is subsequently calculated.

|  | Bin 1 | Bin 2 | Bin 3 | Bin4 | Bin 5 | …. | Bin 74 | Bin 75 | Bin 76 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C_curve | 0 | 1.545 | 3.09 | 4.635 | 6.18 | …. | 112.785 | 114.33 | 115.9 |
| perCap | 0% | 0.65% | 1.30% | 1.95% | 2.60% | …. | 47.58% | 48.2% | 48.9% |

The performance of the compressor for the entire range of its operation is got from combining the performance calculated from the two ranges: no load (Cnl = 0) to just below unload point (Cul) using the load/unload calculations and from unload point (Cul)  to full load (Cfl). The resulting performance profile will be similar to the charts show below.

Example 1

Compressor: 100hp, Single Stage Rotary Screw, 473 cfm

Control Type: Inlet modulation with unloading

Performance Points – All defaults from database at 11.55 atmospheric pressure , 1gal/cfm

Resulting Performance Profile (curve)

Example 1 is modelled in the example file “SS and TS Rotary Screws Model- Hybrid (final)”

Example 2

Compressor: 100hp, Single Stage Rotary Screw, 465 cfm

Control Type: Inlet modulation with unloading

Performance Points – All defaults from database, except UL changed to 10% from default 50%

Resulting Performance Profile (curve)

## Section 4: Finding Performance Curves for Individual Compressor under Start/Stop controls

This algorithm applies to single stage and two stage reciprocating compressors and Two stage lubricant-free rotary screw compressors operating under “start/stop” controls

Applicable Key

| Inputs |  | Units | Imperial Units | Metric Units |
| --- | --- | --- | --- | --- |

| Performance Points – Inputs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Full Load – Airflow | C_fl | acfm | acfm | - |
| Full Load – Power | kW_fl | kW | - | kW |
| Max Full Flow Power | kW_max | kW | - | kW |

| Others |  |  |  |  |
| --- | --- | --- | --- | --- |
| Percent Power Drawn | Perkw | % |  |  |
| Percent Capacity | PerC | % |  |  |
| Max Full Flow Power - Percentage | kWPer_max | % |  |  |
| Power drawn at given point | kwavg | kW | - | kW |
| Airflow at given point | Cusage | acfm | acfm | - |
| Capacity Calculated | Ccalc | kW | - | kW |
| Power Calculated | kwcalc | acfm | acfm | - |

### Step 1: Adjusting Performance Points (inputs)

Perform the appropriate corrections as laid out in the “Pressure and Inlet Correction” document. Summarizing the changes that will affect the modulating controls below.

| Start/Stop | Discharge Pressure | Airflow | Airflow | Power | Power |
| --- | --- | --- | --- | --- | --- |
| Full Load (cut-in) | P_fl | C_fl | Will Change | kW_fl | Will Change |
| Max Full Flow (cut-out) | P_max | C_max | Will Change | kW_max | Will Change |
| No Load (off) | P_nl = 0 | C_nl = 0 | Can't Change | kW_nl = 0 | Will not Change |

### Step 2: Calculating Power/Capacity (Performance profile)

Using Percent Capacity as Input:

kWPer_max = kW_max /kW_fl

CPer = C_usage/C_fl

PerkW =(( kWPer_fl + kWPer_max)/2) * CPer

kW_calc = PerkW * kW_fl

C_calc = C_fl * CPer

Example:

| Inputs |  | Outputs |  |
| --- | --- | --- | --- |
| Variable | Value | Variable | Value |
| kWPer_fl | 1 | PerkW | .206 |
| kWPer_max | 1.05 | kW_calc | 18.4 |
| CPer | .2 | C_calc | 112 |
| kW_fl | 89.5 |  |  |
| C_fl | 560 |  |  |

Using Power as Input:

PerkW = kW_avg/kW_fl

kWPer_max = kW_max /kW_fl

CPer = PerkW/ (( kWPer_fl + kWPer_max)/2)

C_calc = C_fl * CPer

kW_calc = PerkW * kW_fl